Work instruction

[Fit the motor to the gearbox]

| Document no. | [WI-0001] | Revision | [A] |
| --- | --- | --- | --- |
| Product / part no. | [Product name, part number] | Effective date | [date] |
| Station / cell | [Station or cell] | Takt / cycle time | [x] minutes |
| Prepared by | [name] | Approved by | [name, role] |

# Purpose and scope

Purpose: [What this instruction is for, and the result when it is done right.]

Scope: [Which products, models and stations it covers, and which it doesn't.]

# Safety

- [Wear safety glasses and gloves at this station.]

- [Isolate the air supply before working on the press.]

- [Lift parts over 20 kg with the hoist, not by hand.]

- [Report any damaged tool or part to the team leader before carrying on.]

Personal protective equipment: [Safety glasses, cut-resistant gloves, safety boots]

# Tools, equipment and settings

| Tool or equipment | Setting | Note |
| --- | --- | --- |
| [Torque driver] | [2.5 N·m] | [Calibrated, in date] |
| [Hex key] | [4 mm] | [ ] |
| [Press fixture] | [Fixture no. F-12] | [ ] |

# Parts and materials

| Item | Part no. | Description | Qty |
| --- | --- | --- | --- |
| 1 | [PN-1001] | [Gearbox housing] | [1] |
| 2 | [PN-1002] | [Motor] | [1] |
| 3 | [PN-2001] | [M4 × 12 socket cap screw] | [4] |
| 4 | [PN-2002] | [M4 spring washer] | [4] |

# Steps

| Step | Picture | What to do |
| --- | --- | --- |
| 1 | [Picture for step 1: the parts this step uses, highlighted. If you have the CAD model as a STEP file, lineplate.com draws these for you.] | [Place the gearbox housing (1) in the fixture, output shaft facing down.] Key point: [The housing sits flat on all three locating pins.] Check: [Housing doesn't rock.] |
| 2 | [Picture for step 2] | [Offer the motor (2) to the housing, aligning the shaft key with the coupling slot.] Key point: [Don't force it: turn the shaft slightly until the key drops in.] Check: [Motor face is flush with the housing.] |
| 3 | [Picture for step 3] | [Fit the four screws (3) with washers (4), finger tight, then torque in a cross pattern.] Key point: [Torque to 2.5 N·m, in two passes.] Check: [All four screws marked with paint pen after torquing.] |
| 4 | [Picture for step 4] | [Turn the output shaft by hand through two full turns.] Key point: [It turns smoothly, with no tight spots or noise.] Check: [Record pass or fail on the traveller.] |

# Quality checks

- [All fixings torqued and marked.]

- [No damage to the housing or the motor cable.]

- [Output shaft turns freely by hand.]

- [Serial number recorded on the traveller.]

# If something is wrong

[If a check fails: stop, tag the unit with a red tag, put it in the quarantine area and tell the team leader. Don't rework it unless this instruction says how.]

# Revision history

| Rev. | Date | Change | By |
| --- | --- | --- | --- |
| [A] | [date] | [First issue] | [name] |

# Training sign-off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Operator trained | [name] | [signature] | [date] |
| Trainer | [name] | [signature] | [date] |